Content

Setup	3

What is this setup doing?	3

Line 1	4

Line 2	4

⏱ No Prescaler (CS10 = 1)	5

🎧 Noise Canceler Enabled (ICNC1 = 1)	5

🌊 Edge Selection (ICES1 = 0)	5

Line 3	5

⚖ What does the Analog Comparator compare?	6

⚡ What happens in hardware	6

🧠 What we‘ve built	6

How it works in the loop	8

What is the code measuring?	8

⏱ Role of the 16 MHz clock	8

🔢 Converting ticks → time	8

🎯 Frequency formula	9

🤔 Why does the code use 1,600,000,000?	9

🔥 Example	10

🧠 Key insight	10

Why ICR1 and not just TCNT1?	11

🧩 First: what are these registers?	11

🎯 What is Input Capture?	11

⚡ Why not just read TCNT1 in software?	11

🚀 What Input Capture does better	12

🧠 Why this code still uses loops	13

🆚 TCNT1 vs ICR1 summary	13

🎯 Big picture	13

How the analog comparator is wired into the timer capture unit	14

🧠 The Big Idea	14

⚙️ Step 1 — The Analog Comparator	14

🔌 Step 2 — Internal Routing to Timer1	15

⏱ Step 3 — What happens on an edge?	15

🎚 Step 4 — Which edge triggers capture?	16

🧩 Why this is insanely powerful	16

🆚 Without this hardware path	17

🎯 Why this is perfect for FT8	17

📌 Summary	17

// Timer1 configuration (used for frequency measurement)

TCCR1A = 0x00;

TCCR1B = 0x81;        // No prescaler, input capture enabled

ACSR  |= (1 << ACIC); // Analog comparator triggers input capture

Setup

These lines configure Timer1 and the Analog Comparator on the ATmega328P (Arduino Nano) to do very precise frequency or period measurement using hardware — almost no CPU involvement.

What is this setup doing?

We are building a system where:

- Timer1 runs as a high-speed clock.

- The Analog Comparator watches an analog signal.

- When the signal crosses a threshold, hardware:

  - instantly stores the current timer value.

- From the time between two captures → you compute period → frequency.

This is extremely accurate because it avoids software timing delays.

TCCR1A = 0x00;

Line 1

TCCR1A = Timer/Counter1 Control Register A

Setting it to 0x00 clears all bits.

Effect:

| Feature | Result |
| --- | --- |
| Waveform Generation Mode bits (WGM10, WGM11) | 0 → Normal timer mode |  |
| Compare Output (COM1A/COM1B) | Disabled (no PWM on D9/D10) |

👉 Timer1 is just a free-running counter. No PWM, no output compare behavior.

## Line 2

TCCR1B = 0x81;   // No prescaler, input capture enabled

0x81 = 1000 0001 in binary.

TCCR1B = Timer/Counter1 Control Register B

| Bit | Name | Value | Meaning |
| --- | --- | --- | --- |
| 7 | ICNC1 | 1 | Input Capture Noise Canceler ON |
| 6 | ICES1 | 0 | Capture on falling edge |
| 5 | – | 0 | – |
| 4 | WGM13 | 0 | Normal mode |
| 3 | WGM12 | 0 | Normal mode |
| 2 | CS12 | 0 | Prescaler bits |
| 1 | CS11 | 0 | Prescaler bits |
| 0 | CS10 | 1 | Prescaler bits |

## ⏱ No Prescaler (CS10 = 1)

Timer1 runs at full CPU speed:

16 MHz→1 						tick=62.5 ns

That gives very high time resolution, ideal for frequency measurement.

## 🎧 Noise Canceler Enabled (ICNC1 = 1)

The input capture signal must stay stable for 4 clock cycles before triggering.

✔ Reduces false triggers from noise
✖ Adds a tiny delay (negligible for most uses)

## 🌊 Edge Selection (ICES1 = 0)

Capture occurs on a:

HIGH → LOW transition

If you want rising edge instead:

TCCR1B |= (1 << ICES1);

## Line 3

ACSR |= (1 << ACIC); // Analog comparator triggers input capture

ACSR = Analog Comparator Control and Status Register

Bit ACIC = Analog Comparator Input Capture Enable

Normally, Timer1 Input Capture is triggered by the ICP1 pin (D8).

This line reroutes the trigger:

“Use the Analog Comparator output as the capture trigger instead.”

## ⚖ What does the Analog Comparator compare?

| Input | Arduino Pin |
| --- | --- |
| AIN0 (+) | D6 |
| AIN1 (–) | D7 |

When AIN0 crosses AIN1
the comparator output flips → this now triggers Timer1 Input Capture.

## ⚡ What happens in hardware

When the comparator switches:

- Hardware instantly copies TCNT1 → ICR1

- An interrupt can fire (if enabled)

- You subtract two capture values:

Δt=ICR12-ICR11

Then:

f=FCPUΔt

## 🧠 What we‘ve built

We created a hardware measurement chain:

Analog Signal → Comparator → Timer1 Input Capture → Timestamp

✔ No digitalRead()
✔ No polling
✔ No timing jitter from code

This is precision measurement using pure hardware — very efficient and very accurate.

# How it works in the loop

## What is the code measuring?

The analog comparator (bit ACO in ACSR) sees an audio tone (FT8 signal).
The code measures parts of the waveform:

| Variable | What it represents |
| --- | --- |
| d1 | length of one HIGH period |
| d2 | length of HIGH + LOW + next HIGH |
| d2 − d1 | LOW + HIGH = one full period |

So the code extracts the signal period.

## ⏱ Role of the 16 MHz clock

Timer1 runs at the CPU clock = 16 MHz

So:

1 timer tick=116,000,000=62.5 ns

The timer is basically a very fast stopwatch.

## 🔢 Converting ticks → time

If the timer counts N ticks during one signal period:

T=N16,000,000

Here:

N=d2-d1

## 🎯 Frequency formula

Basic physics:

f=1T

Insert T:

f=1(d2-d1)/16,000,000=16,000,000d2-d1

That would give frequency in Hz.

unsigned long codefreq = 1600000000UL / (d2 - d1);

🤔 Why does the code use 1,600,000,000?

Because:

1,600,000,000=16,000,000×100

They scale by ×100 to keep two decimal digits without using floating point.

So:

codefreq=f×100

| Real tone | Stored value |
| --- | --- |
| 1000 Hz | 100000 |
| 2500 Hz | 250000 |

if (codefreq < 350000)   // = 3500 Hz

Later:

## 🔥 Example

Suppose:

d2 - d1 = 8000 ticks

f=16,000,0008000=2000 Hz

Code:

1600000000 / 8000 = 200000

→ 200000 / 100 = 2000 Hz

## 🧠 Key insight

The loop speed does NOT matter here.

Even if the CPU is stuck in those while() loops, Timer1 keeps counting in hardware at 16 MHz. The CPU is just waiting for signal edges; the timer measures time independently.

That’s the big concept:
👉 You measure frequency by measuring time, not by counting loop executions.

# Why ICR1 and not just TCNT1?

## 🧩 First: what are these registers?

| Register | Meaning |
| --- | --- |
| TCNT1 | Timer1 counter (live value, constantly changing) |
| ICR1 | Input Capture Register (a hardware snapshot of TCNT1) |

Think of it like this:

TCNT1 = a running stopwatch
ICR1 = a photo taken of the stopwatch at an exact instant

## 🎯 What is Input Capture?

Timer1 has special hardware:

When a selected signal edge occurs (rising or falling), the hardware:

- Instantly copies TCNT1 → ICR1

- Does this in the same clock cycle

- No software delay

That’s insanely precise — cycle accurate.

## ⚡ Why not just read TCNT1 in software?

Because software is slow and jittery compared to hardware.

In your loop:

while (ACSR & (1 << ACO)) { ... }

The CPU is polling:

- Read register

- Test bit

- Branch

That takes multiple cycles. When the signal edge happens, the CPU might detect it:

- 2 cycles late

- 8 cycles late

- sometimes more

That timing uncertainty = measurement error.

At 16 MHz:

1 cycle = 62.5 ns

10 cycles error = 625 ns

For a 2000 Hz tone:

Period = 500 µs

625 ns error ≈ 0.125% error

That’s already noticeable for precise FSK/FT8.

## 🚀 What Input Capture does better

The analog comparator is internally connected to Timer1 capture.

When the comparator flips:

➡ hardware triggers capture
➡ TCNT1 is frozen into ICR1 instantly

No loop delay. No instruction timing. No jitter.

Precision = ±1 timer tick (62.5 ns). Beautiful.

## 🧠 Why this code still uses loops

The loops are only used to:

- Wait for specific signal transitions

- Define where a period starts/ends

But the actual timestamp comes from hardware capture.

So the CPU is just acting as a traffic cop; the timer hardware does the stopwatch job.

## 🆚 TCNT1 vs ICR1 summary

| Feature | TCNT1 | ICR1 |
| --- | --- | --- |
| Live counter | ✅ | ❌ |
| Snapshot at signal edge | ❌ | ✅ |
| Software latency error | YES | NO |
| Cycle accurate | ❌ | ✅ |
| Best for frequency measurement | 😬 | 🔥 |

## 🎯 Big picture

This system works like this:

- Timer1 runs at 16 MHz

- Comparator sees audio waveform

- Hardware captures timestamps into ICR1

- Code subtracts timestamps

- Time → frequency

This is basically how oscilloscopes and frequency counters work internally — just simpler.

# How the analog comparator is wired into the timer capture unit

This is one of those “hidden superpowers” of the AVR.

We’re talking about:

How the Analog Comparator is internally connected to Timer1 Input Capture

This is pure hardware — no Arduino magic.

## 🧠 The Big Idea

The AVR has an internal signal path:

Analog Comparator Output (ACO)

Timer1 Input Capture Unit

ICR1

So an analog voltage crossing can directly trigger a timer timestamp.

No CPU involved. Zero software delay.

## ⚙️ Step 1 — The Analog Comparator

The comparator compares two voltages:

| Pin | Meaning |
| --- | --- |
| AIN0 | Positive input |  |
| AIN1 | Negative input |  |

If:

AIN0 > AIN1 → ACO = 1

AIN0 < AIN1 → ACO = 0

So your audio signal becomes a digital square wave at the comparator output.

That bit is:

ACSR register → ACO bit

## 🔌 Step 2 — Internal Routing to Timer1

Now comes the cool part.

There is a control bit in ACSR:

ACIC = Analog Comparator Input Capture Enable

When ACIC = 1:

The comparator output is wired internally to Timer1’s Input Capture trigger.

So the timer can react to comparator edges just like it would to an external pin.

## ⏱ Step 3 — What happens on an edge?

When the comparator output changes (depending on edge selection):

- Timer hardware detects edge

- Current value of TCNT1 is copied to ICR1

- In the same clock cycle

- Optional interrupt can fire

This is called an Input Capture Event.

## 🎚 Step 4 — Which edge triggers capture?

Controlled by TCCR1B register:

ICES1 = Input Capture Edge Select

| ICES1 | Trigger |
| --- | --- |
| 1 | Rising edge |
| 0 | Falling edge |

So you can measure:

- High time

- Low time

- Full period

Just by switching edges.

## 🧩 Why this is insanely powerful

You just built:

✔ A zero-crossing detector
✔ A frequency counter
✔ With 62.5 ns resolution
✔ Using only hardware

That’s lab-grade measurement technique.

## 🆚 Without this hardware path

If the comparator wasn’t connected to the timer:

You’d have to:

- Poll ACO in a loop

- Detect edges in software

- Read TCNT1 manually

Result = jitter, missed edges, bad precision.

## 🎯 Why this is perfect for FT8

FT8 tones are close in frequency (only ~6.25 Hz spacing).

To decode them reliably, you need:

- Very precise frequency measurement

- Low jitter

- Stable timing

This hardware capture path makes that possible on an 8-bit MCU 🤯

## 📌 Summary

| Component | Role |
| --- | --- |
| Comparator | Converts audio → digital edges |
| ACIC bit | Routes comparator to timer |
| Timer1 | High-speed time base |
| Input Capture | Hardware timestamp |
| ICR1 | Stores exact edge time |